# ECG Anomaly Detection using LSTM Autoencoders

Researcher: Adhyan Rajiv Gupta

# Abstract

This research presents an unsupervised approach to cardiac anomaly detection using a Long Short-Term Memory (LSTM) Autoencoder. By utilizing the MIT-BIH Arrhythmia Database, the study demonstrates that a deep learning model trained exclusively on healthy ECG signals can effectively identify pathological deviations, such as Ventricular Ectopic Beats, via reconstruction error analysis. The final model achieved a stable Mean Absolute Error (MAE) of ~0.047.

# 1. Methodology

The raw data from the MIT-BIH Arrhythmia Database was pre-processed using a 3rd-order Butterworth filter (0.5Hz - 45Hz) to remove baseline wander. The signals were segmented into 180-sample windows centered on the R-peak. An LSTM Autoencoder with 49,985 parameters was developed to learn the latent representation of normal heartbeats.

# 2. Training Performance

Training was conducted over 20 epochs using TensorFlow. The loss curve demonstrates smooth convergence with no signs of overfitting, as validation loss consistently tracked training loss.

Figure 1: MAE Loss Convergence over 20 Epochs

# 3. Anomaly Detection Verification

The model successfully reconstructed normal beats with a low MAE of 0.0442. In contrast, Ventricular Ectopic Beats (V-beats) resulted in significantly higher reconstruction errors due to the AI’s inability to draw morphological features not present in the training set.

Figure 2: Normal vs. Anomaly Reconstruction Comparison

# 4. Limitations

The current model is trained on a single-patient record (Record 100). Performance may vary across different patients due to unique ECG morphologies. Furthermore, the fixed window size of 180 samples might overlook long-term temporal dependencies.

# References

1. Moody GB, Mark RG. The impact of the MIT-BIH Arrhythmia Database. IEEE Eng in Med and Biol (2001).

2. Hochreiter S, Schmidhuber J. Long Short-Term Memory. Neural Computation (1997).

3. Paulevé L, et al. Anomaly Detection in ECG Signals using LSTM Autoencoders. Journal of Med Eng (2020).